Backplane S/N 004 incoming inspection

- Board had odd odor (burnt-ish) when first removed from static bag. Same odor as S/N 005 – I think it is the chokes.

- Quality of TI power modules significantly better than Murata

- Connections not shiny – appears to be flux residue

- Big diode on TI module installed crooked. Probably OK.

- Capacitor polarities good

- IC orientations good

- Backside diode polarities good

- Drill alignment good

Front side

- NOTE: there are 2 pairs of vias touching each other between J9 and J2. These are same net vias.

- Stickers on board? See images.

- Capacitor polarities good

- Front side diode polarities good

Install R25 ____

Install U6 ____

Install 3 x AD590 ____

Install 2x4 header on back side for J1 ____

Stickers